[image: image38.jpg]S2VRRS .




Gubbs Mass Spec Utilities 2.x.x User Manual
Page 1 of 40

	Gubbs Mass Spec Utilities 2.x.x

GMSU 2.0.34 and earlier

	Supported Platforms


Sciex Analyst 1.3.1 - 1.4.1

	Users Manual

	Gubbs, Inc.

265 Blue Spruce Circle

Alpharetta, GA 30005

P: 770-573-0169

F: 508-453-1338



	gubbs@gubbsinc.com
www.gubbsinc.com




	Table of Contents

Section
Page Number


	51
Introduction


52
High Throughput (HT) vs Low Throughput (LT) Acquisition


63
Console


74
Acquisition Method Generator Utilities


85
PI Method Generator Utility


85.1
Choose a SubProject


85.2
Choose an Assay Request File


85.3
Click on Populate Form


95.4
Enter appropriate information in the Enter New Default Parameters box


95.5
Click on Generate Method


96
MRM Method Generator Utility


106.1
Choose a SubProject


106.2
Choose an Assay Request File


106.3
Modify the Acceptance Criteria as needed


106.4
Click on Populate Form


116.5
Inspect results


116.6
Enter appropriate information in the Enter Default Parameters box


116.7
Click on Generate Methods


127
Modify Methods Utility


127.1
Choose a ‘Populate Form’ option; either ‘Assay Request File’ or ‘All Methods of the chosen Expt Type’.


127.2
Choose an Experiment Type option from the ‘Expt Type’ frame.


127.3
Choose subproject


127.4
Choose Assay Request File from the dropbox


127.5
Enter values for desired variables


127.6
If an Internal Standard is to be added, select the IS checkbox.


137.7
Click on Populate Form to preview data


147.8
Click the ‘Modify Method’ button


148
Automaton File Converter Utility


148.1
Choose an Automaton Text File


148.2
Choose a SubProject


148.3
Click on ‘Populate Form’


168.4
Click on ‘Execute Name Change’ button


179
Batch Queue Submission Utility


179.1
Choose an Assay Request File


179.2
Choose a SubProject


179.3
Choose a Batch Type option from the Batch Type frame.


179.4
Enter Set Name, Identifier #, Batch Owner, and raw data filename (if Batch Type is Final Expt)


179.5
Click on the ‘Populate Form’ button


179.6
Click on Submit Batch


1910
Hepatic Clearance Calculator - Overview


2011
Hepatic Clearance Calculator – Initial Settings


2011.1
Constants - Species


2111.2
Time Point Set


2111.3
Chrom. Method


2111.4
Sample Change Warning boxes


2111.5
Choose Data Access Configuration


2111.6
Choose Data Acquisition Mode


2111.7
Choose a raw data file or raw data file directory


2211.8
Choose a Sample


2312
Hepatic Clearance Calculator – Chromatographic Processing


2312.1
Display chromatography


2512.2
Integration Parameters


2512.3
Manual Integration


2612.4
Manual Peak Setting


2612.5
Manual Peak Retention Time Entering


2612.6
Chromatography Review Dropdown Box


2612.7
Save Chromatogram


2612.8
Batch Accept All Chromatograms


2613
Hepatic Clearance Calculator – Results


2713.1
Overview


2813.2
Use IS Option Frame


2913.3
Plot Option Frame


2913.4
Constants Dropdown Box


3013.5
Exclude Data Points


3013.6
Restore Data Points


3013.7
Comments


3013.8
Review


3113.9
Save Results


3114
Hepatic Clearance Calculator – Reports


3315
Hepatic Clearance Calculator – Additional Functions


3315.1
Accept All Chromatograms


3315.2
Change Data File Path


3315.3
Copy Peak Integrations to Clipboard


3315.4
Generate Reports


3315.5
Peak Area Text File


3415.6
Manually Enter Peak Retention Times


3615.7
Print


3615.8
Enter LT Retention Time – Analyte


3715.9
Enter LT Retention Time – Int Std


3715.10
Display Multiple Results


3716
Permeability Calculator


4017
References






	Table of Figures
Figure
Page Number


	6Figure 1
GMSU Console


7Figure 2
Acquisition Method Generator Utilities


8Figure 3
PI Method Generator startup screen


9Figure 4
After clicking the Populate Form button


10Figure 5
MRM Generator Utility


11Figure 6
After clicking Populate Form button


12Figure 7
Modify Methods Utility


13Figure 8
Choose Internal Standard Method File Source


13Figure 9
Populated Modify Methods Window


14Figure 10
Results of ‘Modify Method’ button click


15Figure 11
Automaton File Converter Utility


16Figure 12
Changed Automaton Filenames


18Figure 13
Populate Form – PI/MRM Experiment


19Figure 14
Populate Form – Final Expt Experiment


20Figure 15
Hepatic Clearance Calculator


22Figure 16
Loaded raw data file data


24Figure 17
Click on Peak 1.40 of Analyte Retention Time/Peak Area table grid (with ‘Synchronize Analyte w/IS selected


25Figure 18
Manual Integration


27Figure 19
Hepatic Clearance Results Pane Overview


28Figure 20
Use IS Frame Options


29Figure 21
Plot Frame Options


29Figure 22
Species Constants Settings


30Figure 23
Excluding Points


31Figure 24
GMSU Console


32Figure 25
Hepatic Clearance Calculator Reports Module


33Figure 26
Menu Items


34Figure 27
Manually Enter Peak Retention Times


35Figure 28
Use peaks from an existing sample


36Figure 29
Use peaks from an existing chromatographic method


37Figure 30
Permeability Calculator – Chromatography Tab


38Figure 31
Permeability Calculator – Results Tab


39Figure 32
Permeability Calculator – Calibration Tab






1 Introduction

Gubbs Mass Spec Utilities (GMSU) is a suite of utilities that enhance the ability of scientists to perform high volume actions in mass spectrometer data acquisition systems. This is useful in laboratories engaged in high-throughput studies.

GMSU uses either Microsoft® (MS) Access or MS SQL Server as a datastore. MS Access may be used to quickly setup a GMSU instance for user testing or production use. MS SQL Server may be used if a more secure environment is desired. The MS Access database can be directly upgraded to SQL Server without loss of data.

GMSU may be used in a multi-user environment. The GMSU database and acquired mass spectrometer raw data files may be stored in a common location accessible to others. Multiple installations of GMSU may be configured to use the database and access the raw data files located in the common location. This allows users in the analytical department to use GMSU to generate and optimize chromatographic integration acquired data. Once the chromatographic integration data is approved, users in the pharmacokinetics department may be notified and can immediately generate pharmacokinetic data.

GMSU data are stored in a database (either MS Access or MS SQL Server). This data may be accessed or generated from multiple installations of GMSU.

System requirements and installation procedures, please refer to the Gubbs Mass Spec Installation and Administration Manual.

2 High Throughput (HT) vs Low Throughput (LT) Acquisition

One of the salient features of GMSU is that it can process HT acquired data as well as LT acquired data. Typically, mass spectrometry data acquisition systems require that compound(s) of interest occur once in a chromatographic analysis. Therefore, in order to analyze a 12-point assay, scientists had to acquire twelve separate ‘samples’ within a data file. This is referred to as LT acquisition in the GMSU system.

In HT acquisition, users open the data acquisition window for as long as needed and inject and acquire all assay time points in a single chromatographic run. The benefits of HT acquisition 1 - 4 include:

· The ability to view all chromatographic peaks simultaneously to check for possible instrumental or sample handling problems and/or trends

· When combined with multiple-column switching techniques, increases throughput by decreasing the time between sample injections and minimizes communication problems that can occur between instrument and data acquisition system.

3 Console
The GMSU Console is the module from which users navigate to other modules. The Console contents may differ slightly from what is shown.
[image: image1.png]3 GMsU Console

Gubbs™Mass Spec Utilities

- Home Acquisition Method Generator Ut
Batch Queue Submission Utility

Hepatic Clearance Calculator

Permeability Calculator

Configuration Ut





Figure 1
GMSU Console

Users click on the appropriate buttons either to open a utility or to navigate to an additional hierarchical level.

4 Acquisition Method Generator Utilities

The Acquisition Method Generator Utilities are a suite of utilities that are focused on specific tasks involved in acquisition method generation. The available Method Generator Utilities are described as follows:

[image: image2.png]3 GMsU Console

Gubbs™Mass Spec Utilities

- Home
” Acquisition
Method

Generator
Utilities

PI Method Generator Utility
MRM Method Generator Utility

Modify Methods Utility

Automaton File Converter

Go Back





Figure 2
Acquisition Method Generator Utilities

5 PI Method Generator Utility

The PI Method Generator Utility generates Product Ion (PI) experiments configured with one positive ion (M+1) experiment and one negative ion (M-1) experiment. Methods are generated based on data contained in a selected Assay Request file that contains analyte name and its corresponding molecular weight. The following describes the procedure for using the PI Method Generator.
[image: image3.png]B3 Product lon Method Generator

Product Ion Method Generator

choose SubProject

= Method does not exit.
== If method exits, it wil be overwritten when executing Generate Method.

Existing Parameter Values *
Dwell Coll

Sample 1D Proposed Method File Name **





Figure 3
PI Method Generator startup screen
5.1 Choose a SubProject

5.2 Choose an Assay Request File

Note: Users may view the Assay Request File by clicking the View File button

5.3 Click on Populate Form

The form is populated with information.

· If a PI acquisition method (.dam) file already exists, its parameter information will be displayed.

· If a PI .dam does not exist, the parameter information columns will contain asterisks
· If the user wishes to populate the form with information from a different Assay Request file, the Clear Form button must first be clicked.

[image: image4.png]B3 Product lon Method Generator

Product Ion Method Generator

choose SubProject

Choose Assay Request File

View File

= Method does not exit.

== If method exits, it wil be overwritten when executing Generate Method.

sample D

Proposed Method File Name **

Existing Parameter Values *

Mw

Coll.

ML00304936-001-A

ML00304936-001-A_PLdam

303.203

ML00275841-001-A

ML00275841-001-A_PLdam

345.421

ML00059375-489-A

ML00059375-489-A_PLdam

289375

ML00028428-001-F

ML00028428-001-F_PLdam

425.920

ML00013547-001-D

ML00013547-001-D_PLdam

324424

MLO0059463-001-C

ML00059463-001-C_PLdam

285.362

ML00008113-001-C

ML00008113-001-C_PLdam

183.185

ML00026714-001-F

ML00028714-001-F_PLdam

213.664

ML00059423-001-C

ML00059423-001-C_PLdam

230263

ML00028668-001-G

ML00028668-001-G_PLdam

277.266

ML00028262-001F

ML00028262-001-F_PLdam

248326

ML00028208-001-D

ML00028208-001-D_PLdam

234343

ML00028318-001-F

ML00028318-001-F_PLdam

253.260

MLO0008818-003-C

ML00008818-003-C_PLdam

339301

ML00059447-004-A

ML00059447-004-A_PLdam

334304





Figure 4
After clicking the Populate Form button
5.4 Enter appropriate information in the Enter New Default Parameters box

5.5 Click on Generate Method

PI acquisition methods with the Proposed Method File Name will be created that have parameters shown in the Default Parameters box.
If the method already exists, that method will be overwritten.
6 MRM Method Generator Utility
The Multiple Reaction Monitoring (MRM) Method Generator utility performs several functions:

· Allows users to quickly view Total Ion Current (TIC) and mass spectra contained in acquired .wiff files that contain an M+1 positive ion experiment and an M-1 negative ion PI experiment.

· Determines the most appropriate MRM experiment to generate based on user-configurable criteria.
· Flags .wiff files whose experiments do not meet criteria.
· Generates MRM methods for non-flagged experiments using parameters set by the user.
The utility will determine and record the most abundant daughter ions obtained from the M+1 positive ion experiment and M-1 negative ion PI experiment. The utility will then recommend the most appropriate MRM experiment to generate based on the daughter ion that is most abundant. Users may manually override any decision made by the utility.

The following describes the procedure for using the MRM Method Generator utility.

[image: image5.png]3 MRM Method Generator Utility.

MRM Method Generator Utility
orets | coerom

Choose Assay Request File _View File

Duell Colision
View in Analyst
1Csyne —_—
250 30 [iCome Manual Accept

Manual Flag
Methods wil ot be genereted for lagged anabes — |

Parent .wiff File/
. Proposed MRM Acq. Method





Figure 5
MRM Generator Utility

6.1 Choose a SubProject

6.2 Choose an Assay Request File

Note: Users may view the Assay Request File by clicking the View File button

6.3 Modify the Acceptance Criteria as needed
· Min Daughter m/z: The minimum daughter ion m/z that will be considered when evaluating mass spectra

· Min Daughter Intensity: Ions must be greater than this value in order to be considered as acceptable

6.4 Click on Populate Form

If the user wishes to populate the form with information from a different Assay Request file, the Clear Form button must first be clicked.

[image: image6.png]MRM Method Generator Utility Populate | Generate rorm
Form Methods | <2
Choose Assay Request Fle _View File | Choose SubProject
Enter Default Parameters Enter Acceptance Criteria
Dwel Colision Hin Hin -
Time  Duration  Energy Daughter  Daughter View in Analyst
(mseq) (mim) (V) LCsyne miz__Intensity T
B50 B 30 [iCome 5] 70 20,000 Manual Accept
*Methods wil ot be generaled fo Flagged antes
*Neither Neg or Pos scan met Minimum Daughter lon ntensiy citeris. Manual Flag
Daughter
Parent wiff File/ Parent Daughter  Ion
Proposed MRM Acq. Method Ton Ion _Intensity _Accept _Flag*

B 1100008180 001,
ML00008818-003-C_MRM.dam MNeg| 338.39 | 16585 2,834

2 [ML00059447-004-A_P1 001wt Pos| 33539 | 0.00 o = Flag
[ML00059447-004-A_MRM.dam MNeg| 33339 | 17258 2,834

3 [ML00028739-003-C PI_001.wif Pos| 372.87 | 17503 | 405262] pos
ML00028739-003-C_MRM.dam Neg| 370.87 | 12035 2,834

4 [ML00275611-001-A P 001wt Pos| 154.14 | 13578 | 22,672 == Flag
[ML00275611-001-A MRM.dam MNeg| 152.14 | 108.09 | 14,170|

5 [ML00002014-001-G_P1 001wt Pos| 27135 | 0.00 o = Fbg
ML00002014-001-6_MRM.dam Neg| 260.35 | 170.02 5. 668]

6 |[ML00059374-046-A_P1 001wt Pos| 37501 | 201.96 | 906,880] Pos
[ML00059374-046-A_MRM.dam Neg| 37391 | 0.00 o

7 [ML00028322-003-E PI_001.wif Pos| 45561 | 165.00 | 600,808] Pos
ML00028322-003-E_MRM.dam Neg| 45361 | 34015 2,834

8 [ML00028643-001-F P1 001wt Pos| 24370 | 0.00 o = Flag
[ML00028643-001-F MRM.dam Neg| 24170 | 0.00 o

9 [ML00028752-001-D_P1 001wt Pos| 27031 | 18005 | 2p672] = Fbg
ML00028752-001-D_MRM.dam Neg| 27731 | 0.0 0

10 [ML00028331-003E PI_001.wif. Pos| 480.53 | 314.00 | 651,820 Pos
[ML00028331-003-E_MRM.dam MNeg| 478.53 | 35571 2,834]

W TIC of +hiS2 (340.39)CE (20)

11087

1.0087.

0020

co0ee

<0020

Intensity, cps

200e0
0.0

Time, min

W L2 (240.29) CE (305 Bxp 1.1

s7es
s0s5.

s0s
2085

2085

Intensity, cps

1025

sz (2089 B2 1244 mi

208

e0s

<08

Intensity, cps

2086

sese

750 260

.

Max. 1,227 cos

02 04 06 U8 10 12 14 10 13 20

Max. 5725 cos

ESEN

e





Figure 6
After clicking Populate Form button

6.5 Inspect results
Users should inspect the results by ensuring that the Accepted Pos or Neg ion is the desired daughter ion to be used when generating the MRM method. Users can view the mass spectra for each analyte by selecting it in the table.

Users have perform the following functions

· Manual Accept: Users may manually accept a Positive or Negative ion by selecting the appropriate row and clicking on the Manual Accept button.

· Manual Flag: Users may manually flag an analyte by selecting either the Positive or Negative row of an analyte and clicking on Manual Flag

· View in native data acquisition system: If desired, users may view the data in the native system by selecting the analyte and clicking on View.
6.6 Enter appropriate information in the Enter Default Parameters box

6.7 Click on Generate Methods

MRM acquisition methods with the Proposed Method File Name and the Accepted positive or negative transitions will be created.

If the method already exists, that method will be overwritten.

7 Modify Methods Utility

The Modify Methods Utility performs several functions:

· Allows users to modify common method variables of existing methods in batch mode based on a user-specified text/Excel file or all methods within a directory.

· Allows users to add an Internal Standard (second transition in a period) to existing methods (form MRM methods only)

The following describes the procedure for using the MRM Method Generator utility.

[image: image7.png]A Modify Methods Utility

Modify Methods Utility

choose SubProject ~ AlMethods of the
———— " ChosenEupt Type

@ Assay Request il

Evpt Type
cn

& MRM

Existing Parameter Values

Method File Dwell [ Dur- | Coll. |Source|
(blank methods ignored) Time | ation |Energy| Temp





Figure 7
Modify Methods Utility
7.1 Choose a ‘Populate Form’ option; either ‘Assay Request File’ or ‘All Methods of the chosen Expt Type’.

If ‘All Methods of the chosen Expt Type’ is chosen, then the ‘Choose Assay Request File’ dropbox becomes disabled and is unneeded.

7.2 Choose an Experiment Type option from the ‘Expt Type’ frame.

If Product Ion (PI) is chosen, then ‘IS’ checkbox is hidden and is not available.

7.3 Choose subproject

7.4 Choose Assay Request File from the dropbox
7.5 Enter values for desired variables

7.6 If an Internal Standard is to be added, select the IS checkbox.

Two dropdown boxes will appear for Internal Standard method file source configuration.

Both dropdowns must be completed (see Figure 8).
[image: image8.png]A Modify Methods Utility

Modify Methods Utility

choose SubProject

@ Assay Request il

~ AllMethods of the
chosen Expl Tope
[cr\analyst data\Projects\AA\AA-ARA

Evpt Type

Choose Assay RequestFile _View File P

CompoundList_0059.xIs =l @ RN

Enter New Default Parameters.

Dwell  Duration Collision  Source
Time (min) Energy Temp
A A A A

Method File
(blank methods ignored)





Figure 8
Choose Internal Standard Method File Source

7.7 Click on Populate Form to preview data

A populated window is displayed (see Figure 9). 

If a method of the type chosen is not found, then the ‘Method File’ field for that analyte will be blank and ignored when the ‘Modify Method’ button is clicked.

The existing parameters of the existing methods are shown in ‘Existing Parameter Values’ columns.
If the method already contains an internal standard, it will be replaced with the Internal Standard information chosen when the ‘Modify Method’ button is clicked.

[image: image9.png]A Modify Methods Utility

Modify Methods Utility

Enter New Default Parameters.
Duel Dwation Colison  Source

Populate
choose subProject Form

Modify
Choose Assay Request File _ View File Method

Clear Form

@ Assay Request il

£~ AllMethads of the
" chosen Expt Type

Evpt Type
cn
& MRM

(min) Temp LCsync 13

\_1 L] \_1 L] w B 7 e E
Existing Parameter Values
Method File Contains | Dwell | Dur- | Coll. |Source

L Sample 1D (blank methods ignored) Polarity | 15 | Time | ation |Energy| Temp Lcsync
] pH_55 PH_55_MRM.dam Pos Tue | 70 | 16 | 47 | a00 |icsync
2] pH_65 PH_65_MRM.dam Pos True | 70 | 16 | 47 | a00 |icsync
3 pH_74 PH_74_MRM.dam Pos True | 70 | 16 | 47 | a00 [icsync





Figure 9
Populated Modify Methods Window

7.8 Click the ‘Modify Method’ button

The method parameters are updated accordingly and are reflected in the table.

Methods that are identified as Pos have positive Internal Standard method transitions added; while methods that are identified as Neg have negative Internal Standard method transitions added.

[image: image10.png]A Modify Methods Utility

Modify Methods Utility

choose SubProject

Choose Assay RequestFile _ ViewFile

Enter New Default Parameters.
Duel Dwation Colison  Source

|

Populate | @ Assa RequestFie
Form | - AlMethads of he
© ChasenExpt Type
Modify ExptTope
Method P
@ MRM
Clear Form

Pos 15 [22Prec_FinaMRM Pos.dem  ~/

(....) Izmv LCsync 13
m S| ® negrs[sPrc FrabRi Negdam 7
Existing Parameter Values
Method File Dwell [ Dur- | Coll. |Source|

L Sample 1D (blank methods ignored) Time | ation |Energy| Temp Lcsync
] pH_55 PH_55_MRM.dam Pos Tue | 70 | 3 | 47 | a00 |icsync
2] pH_65 PH_65_MRM.dam Pos Tue | 70 | 3 | 47 | a00 |icsync
2] pH_74 PH_74_MRM.dam Pos True | 70 | 3 | 47 | a00 [icsync





Figure 10
Results of ‘Modify Method’ button click
8 Automaton File Converter Utility

Utility for Sciex equipment only.
The Sciex Automaton application allows users to generate autotuned acquisition method files in batch mode based on a user inputted text file. Unfortunately, the Automaton utility generates files with filenames that are a combination of previously-mentioned text filename and sample order. Therefore, when a user is searching for method files to configure in an instrumental sequence, the user needs to interpret with a cross-reference table the name of the file to determine which method file is associated with a given analyte.

It would be beneficial if the method file names included the analyte name, thus eliminating any ambiguity involved in determining the identity of the method file.

The Automaton File Converter renames Automaton-generated files to a filename that includes the analyte name.
The following describes the procedure for using the Automaton File Converter Utility.

8.1 Choose an Automaton Text File

8.2 Choose a SubProject

8.3 Click on ‘Populate Form’
The form is populated according to the following rules (see Figure 11).
If an Automaton-generated method file that corresponds with the chosen Automaton text file exists, then:

· the ‘Automaton.dam’ column row is populated with the Automaton-generated method filename

· the ‘Automaton Exists’ column row is populated with ‘Yes’

· the ‘Proposed .dam’ column row is populated with the new proposed filename

If an Automaton-generated method file that corresponds with the chosen Automaton text file does not exist, then

· the ‘Automaton.dam’ column row is left blank
· the ‘Automaton Exists’ column row is populated with ‘No’.

· the ‘Proposed .dam’ column row is populated with the new proposed filename, though this will have no effect on the next actions.

[image: image11.png]i il populate Execute
Automaton File Converter Utility Bopuiat ‘ et | dearrom
Choose Automaton Text File _(View File || Choose SubProject
=] =
Automaton Name a
Automaton .dam Exists Proposed .dam Changed |
1 (6C25 plate) MRM.dam es| Analyte-001-001-A_MRM.dam
2 [2 (€25 plate) wRM dam Yes | Analyte-002-001-4_WRM dam
3 |3 (5C25 plate) WRM dam Yes |Analyte-003-485-4_WRM dam
4[4 (5C25 plate) WRM dam Yes |Analyte-004-D01F _MRM.dam
5 [5 (€25 plate) WRM dam Yes | Analyte-005-001-D_WRM dam
6 [6 (5C25 plate) WRM dam Yes | Analyte-006-001-C_WRM dam
7 |7 (€25 plate)_wRw dam Yes |Analyte-007-001-C_WRM dam
6 |8 (5C25 plate)_WRM dam Yes |Analyte-008-001-F_MRb.dam
5[5 (5C25 plate)_wRM dam Yes |Analyte-006-001-C_WRM dam
10[10 (625 plats) MRM.darn Yes |Analyte-010-001-6_WRM dam
11[11 (625 plats) MRM.darm Yes |Analyte-011-001-F MRM.dam
12[12 (6C25 plats) MRM.darm Yes [Analyte-012-001-D_WRM dam
13[13 (6C25 plate) MRM.darm Yes |Analyte-013-001-F MRM.dam
14[14 (625 plate) MRM.darm Yes |Analyte-014-003-C_WRM dam
15[15 (6C25 plats) MRM.darm Yes [Analyte-015-004-4_WMRM dam
16[16 (6C25 plats) MRM.darm Yes |Analyte-016-003-C_WRM dam
17[17 (6C25 plate) MRM.darm Yes |Analyte-017-001-4 WM dam
18[18 (6C25 plats) MRM.darm Yes |Analyte-016-001-6_WRM dam
15[19 (625 plats) MRM.darm Yes |Analyte-019-046-4 WM dam
20[20 (6C25 plate) MRM.dam Yes |Analyte-020-003-E MRM.dam
2121 (6C25 plate) MRM.dam Yes |Analyte-021-001F MRb.dam
22[22 (625 plate) MRM.dam Yes |Analyte-022-001-D_WRM dam
23[23 (625 plate) MRM.dam Yes |Analyte-023-003-E MRM.dam
2424 (6C25 plate) MRM.dam Yes |Analyte-024-035-4_WMRM dam
25[25 (625 plate) MRM.dam Yes |Analyte-025-003-C_WRM dam
26[26 (6C25 plate) MRM.dam Yes |Analyte-026-003-F MRb.dam
27[27 (625 plate) Rbdam Yes |Analyte-027-001-D_WRM dam 1





Figure 11
Automaton File Converter Utility

8.4 Click on ‘Execute Name Change’ button

When the ‘Execute Name Change’ button is clicked, only the rows that have an entry in the ‘Automaton .dam’ column are processed.

The application changes the filename of the method file, then checks the directory to ensure that the filename has been changed.

When the name change has been verified, ‘Yes’ will be entered in the appropriate row of the ‘Name Changed’ column and the ‘Automaton .dam’ column row value will be cleared (see Figure 12).

[image: image12.png]i il populate Execute
Automaton File Converter Utility Bopuiat ‘ et | dearrom
Choose Automaton Text File _(View File || Choose SubProject
B ]
Automaton Name a
Automaton .dam Exists Proposed .dam Changed |
No Analyte-001-001-A_MRM.dam Yes
2 o rialyte-002-001-A_bRb.dam Yes
3 o rialyte-003-469-A_bRb.dam Yes
4 o rialyte-004-001-F_MVRM.darm Yes
5 o rialyte-005-001-D_MRb.dam Yes
3 o rialyte-006-001-C_bRb.dam Yes
7 o rialyte-007-001-C_bRbdam Yes
5 o rialyte-008-001-F_MRM.darn Yes
9 o rialyte-008-001-C_MRb.dam Yes
10 o rialyte-010-001-G_MRb.dam Yes
1 o rialyte-011-001-F_MVRM.darm Yes
2 o rialyte-012-001-D_MRb.dam Yes
3 o rialyte-013-001-F_MRM.darm Yes
4 o rialyte-014-003-C_bRb.dam Yes
15 o rialyte-015-004-A_bRb.dam Yes
16 o rialyte-016-003-C_bRb.dam Yes
I o rialyte-017-001-A_bRb.dam Yes
1 o rialyte-016-001-G_MRb.dam Yes
19 o rialyte-019-046-A_bRb.dam Yes
o) o rialyte-020-003-E_MRM.darm Yes
21] o rialyte-021-001-F_MVRM.darn Yes
2 o rialyte-022-001-D_MRb.dam Yes
ES o rialyte-023-003-E_VRM.darm Yes
24 o rialyte-024-035-A_bRb.dam Yes
ES o rialyte-025-003-C_MRbdam Yes
E3 o rialyte-026-003-F_MRM.darm Yes
27| o rialyte-027-001-D_MRb.dam Yes 1





Figure 12
Changed Automaton Filenames

9 Batch Queue Submission Utility
The Batch Queue Submission Utility allows users to automatically create a batch sequence that whose samples contain existing unique data acquisition methods (.dam) and collect the data into a single raw data file (.wiff). This is useful in a high throughput environment in which tens to hundreds of samples are acquired in one analytical sequence. The Sciex alternative to this utility requires manually entering/choosing the correct method file for each sample.
After preparing a batch and before submitting, the user may save the batch by clicking the ‘Save the Batch’ button. As expected, the user may load this batch again at a later time.
The following describes the procedure for using the Batch Queue Submission Utility.

9.1 Choose an Assay Request File

9.2 Choose a SubProject

9.3 Choose a Batch Type option from the Batch Type frame.

9.4 Enter Set Name, Identifier #, Batch Owner, and raw data filename (if Batch Type is Final Expt)
9.5 Click on the ‘Populate Form’ button

The ‘Method File’ column will be filled with the appropriate existing method files based on the contents of the Assay Request File. If the expected method file does not exist, the ‘Method File’ column row entry will be blank.

If the Batch Type was PI or MRM, then the ‘.wiff File Name’ column row entry will be created corresponding to the sample name (see Figure 13). If the Batch Type was ‘Final Expt’, then the ‘.wiff File Name’ column will be named according to the user entry described in Section 9.4 (see Figure 14).

9.6 Click on Submit Batch

When the user is ready to submit the batch to the queue, the ‘Submit Batch’ button is pushed.

Note: Rows that have blank entries in the ‘Method File’ column will be ignored and not submitted.

[image: image13.jpg]Batch Queue Subm

Batch Queue Submission Utility

{Load a Saved Batch.

Save the Batch... |

Enter Default Parameters

i Fle
Identifier # _Batch Owner
Method File
Sample 1D (blank methods ignored) wWiff Fle Name
136-001-A 136-001-A PLdam 36-001-4_PL_0OL wif
10014 1-001-A PLdam 41-D01-A_PL_00L wif
75-489-4 5-485-A_PLdam 75-483-A_PLOOLwif
128-001F 28-00LF PLdam 28-001F 1 O0Lwiff
#7-001-D 47-001-D PLdam 47-D01-D_ P 00L wif
163-001-C 63-001-C_PLdam 63-001-C_P1_00L wif
13-001-C 13-001-C_PLdam 13-001-C_PI ODLwiff
"14-001F 14-001-F PLdam 14-001-F_PL00Lwif
123-001-C 23-001-C_PLdam 23-001-C_P1_00Lwif
166-001-G £E-001-G PLdam 68-001-G_P_00L wif
2-001F 52-001F _PLdam 62-001F_PI_ODLwiff
108-001-D 0B-001-D_PLdarm 08-001-D_P1_00Lwif
J18-001F {E-D01F _PLdam 16-001-F_PL0OLwif
}18-003-C 5-003-C_PLdam 16-003-C_PI ODLwiff
147-004-4 47-004-4 PLdam 47-D04-_P1L_OOL wif
39-003-C 39-003-C_PLdam 39-003-C_P1_00L wif
10014 11-001-A PLdam 11-001-4_PI_ODLwiff
J4-001-G 14-001-G PLdam 14-001-G_PI_ODLwiff
740464 4-046-A_PLdam 74046 PLOOLwif
122:005E 2-003-E PLdam 22-003€ PI_O0Lwiff
#3-001F A3-00LF _PLdam 43-00LF I ODLwiff
52:001-D 52-001-D_PLdarm 52-001-D_ P 00Lwif

131-003-E

G1003-E PlLdam

31-003-E_P1_00Lwiff





Figure 13
Populate Form – PI/MRM Experiment

[image: image14.jpg]Batch Queue Subm

Batch Queue Submission Utility

{Load a Saved Batch.

Save the Batch... |

Enter Default Parameters

wiff File
Identifier # _Batch Ouwner wiff File Name
o0t Gy | ot
Method File
Sample 1D (blank methods ignored) wiff File Name

136-001-A 36-001-4_MRW.dam 20050318 wiff
WI001-A 41-001-4_MRW.dam 20050318 wiff
¥75-489-A 75-489-5_MRW.dam 20050318 wiff
128-001-F 26-001-F_WRM.darm 20050318 wiff
¥7-001-D 47-001-D_MRW.dam 20050318 wiff
163-001-C 63-001-C_MRWM.dam 20050318 wiff
13-001-C 13-001-C_WMRM.darm 20050318 wiff
14-001F 14-D01F_MRM.dam 20050318 wiff
123-001-C 23-001-C_MRW.dam 20050318 wiff
001G 66-001-G_MRW.dam 20050318 wiff
%2-001F 62-001-F_WVRM.darm 20050318 wiff
ME-001-D 08-001-D_MRW.dam 20050318 wiff
ne-001F 18-001F_MRM.dam 20050318 wiff
N8-003-C 18-003-C_MRM.darm 20050318 wiff
147-004-A 47-004-A_MRW.dam 20050318 wiff
35-003-C 39-003-C_MRW.dam 20050318 wiff
$11-001- 11-001-A_VRM.darm 20050318 wiff
114-001-G 14-001-G_VRM darm 20050318 wiff
74046 74-046-5_MRW.dam 20050318 wiff
22-003E 22-003-E_WRM.darm 20050318 wiff
WI00LF 43-001-F_WRM.darm 20050318 wiff
52-001-D 52-001-D_MRW.dam 20050318 wiff

131-003-E

31-003-E_MRM.dam

20050318, wiff





Figure 14
Populate Form – Final Expt Experiment
10 Hepatic Clearance Calculator - Overview
The Hepatic Clearance Calculator consists of three panes (see Figure Figure 15). Users use the top pane (Section 11) to choose data files, configure default constants, choose samples, and choose Warning settings. Users use the bottom left pane (Section 12) to review and optimize chromatographic integration. Users use the bottom right pane (Section 13) to process and optimize Hepatic Clearance results.
In addition, Hepatic Clearance Calculator allows users to generate reports (Section 0) and perform additional functions (Section 15, e.g. copy peak integration data to the clipboard).
The following sections describe the procedures for using the Hepatic Clearance Calculator.

11 Hepatic Clearance Calculator – Initial Settings

The initial state of the Hepatic Clearance Calculator is a screen lacking any data (Figure Figure 15). Users first must set default constants, then choose a data file for processing. The following describes the steps for performing these actions:
[image: image15.png]=
Hepatic Clearance Calculator

Chosse Data Acaisiion Mode:
 Low Thioughput Acaisiion
@ High Thoughput Acquisiion

Chaase Data Access Carfiguraton.
@ Data based on a single data e
€ Data based an data fles witin a drectory

. [

Choose a Sample

Expand

Enable Sample Change Warring
F Chromatoaraphy.
(& Enable Sampe Change Warring

Smooths

= =
S —
o
w
w

@ PainttaFoint
 Dioptobaseine | [

Chromatogr. Review

=

= Peaks:

Analyte
Normaize Y
@ Yes
c o
0

o

eo

w0

Synchvonize
analyte wis

Int Std
Normaize Y
@ ves
)
[ax'

Renute
5 Cancel | Exit
Analyte: Time Point Set: | Microsome: -
|Chrom. Method: | [None] ~
Results Pane Constants
Comments (<250 characters) Spedes: ~
‘g of Liver Weight/Kg Body W]

Results Review

[ =] Resuls

=y

I~ Resuts Saved and Locked

-
100
20
eo
w0
20
o0l
¢z : &
Plot:
Reset || G ves || @ ave
o  Reps

mg Microsoml Protein/ml Incubation
mg Microsoml Protein/g Liver Wt

Hepatic Blood Fow:

172

Intrinsic Hep. Clearance (Lr/ko):
Predicted Hep. Clearance (Ljr/ko):

e

LastPoint Remaining (%):

Chrom Data Used for Results





Figure 15
Hepatic Clearance Calculator
11.1 Constants - Species
Ensure the appropriate species-specific experiment constant set is displayed here. Constants Sets are created and configured in the Hepatic Clearance Calculator Configuration Settings tab of the Configuration Utility. The default constant is retrieved from the Hepatic Clearance Calculator Configuration Settings in the Configuration Utility.
Note: If a dataset contains mixed species experiments, the user later can change the assigned constants by changing the contents of the Species: dropdown box after the dataset has been loaded.

11.2 Time Point Set

The Time Point Set contains assay time points to be applied to the chosen data file.

Ensure an appropriate Time Point Set is displayed here. Time Point Sets are created and configured in the Hepatic Clearance Calculator Configuration Settings tab of the Configuration Utility. The default Time Point Set is retrieved from the Hepatic Clearance Calculator Configuration Settings in the Configuration Utility.

11.3 Chrom. Method

Note: This field is optional for HT acquisition and not applicable to LT acquisition.

The Chrom. Method (chromatography method) is a chromatographic method containing expected chromatographic retention times to be applied to the chosen data file. This method may be applied to the data file if the GMSU automatic peak identification algorithm is not successful (for, e.g., data files with low signal).

Ensure an appropriate Time Point Set is displayed here ([None] if not used). The default Chrom. Method is retrieved from the Hepatic Clearance Calculator Configuration Settings in the Configuration Utility. Chrom. Methods are created and configured in the Hepatic Clearance Module via the Menu – Manually Enter Peak Retention Times… feature (see Section ).
11.4 Sample Change Warning boxes
If the user attempts to select a new sample, but hasn’t saved the Chromatograms or Results, two warnings are displayed. These warnings may become annoying if initial data review is all that is desired. By deselecting one or both of these warnings, the warning messages will be disabled.
11.5 Choose Data Access Configuration
Users may choose to load a single .wiff file or all .wiff files within a directory.

The default setting for this frame is configured in the Hepatic Clearance Calculator Configuration Settings in the Configuration Utility. If desired, an administrator can make this option frame invisible.
11.6 Choose Data Acquisition Mode
Users must select whether the data file being loaded is LT or HT. If the data file does not exist in the database, users will be prompted to assign the data as LT or HT.

The default setting for this frame is configured in the Hepatic Clearance Calculator Configuration Settings in the Configuration Utility. If desired, an administrator can make this option frame invisible.

If the frame is invisible, new data will be processed automatically according to the LT or HT setting and users will not be prompted to assign data.

11.7 Choose a raw data file or raw data file directory
Choose the Browse… button to browse to a raw data file or raw data file directory
Data will be loaded into the ‘Choose a Sample’ table grid and the first sample will be processed. The resulting window is shown in Figure 16.
[image: image16.png]Hepat lculator
=
Hepatic Clearance Calculator Choose a Sample . - Expand
Choase Data Access Confiuration. Choose Data Acauision Mode: T ebenide Tobutanide D017 | et
@ Data based on a single data e € Low Throughput Acquisiion Mephenytoin Mephenytoin 1 08,2006 17 (¥ (et Demplo han
" Data based on dala fes within adiectory | | & High Thoughput Acauisiion Destomethorphen [ Destiomethorphon _[Jul08.2006 18 | gpatis Samele Change Woaring
Teststerone Teststerone 2w 082006 18 |V Resus
[Er\- Proicts 000041200607 10_FiepGHTPIb pata 20060708 Kim Vmax wit | @im > Cance xi
B Smots N AT [ Chromatogr. Review Analyte: Tme ot st [Fcrosome - |
=pesks: enacetn
c [NoFs [ frenaces chvom, wethod: [Tiene] -
i
W C or R 2 pai 19111111 Mo 108t cen. Analyte [0 IOV
Normaize Y| 1.5 7080.71 Results Pane Constants
232 (808 700 @res 230 @ Comments (<250 characters) | Species: [Ratigosame -
To0es 2 2 - c o 306 el o s
so0000] 128 0 || 3er erasas ‘g ofLver Weight/Kg Body W =
g a5 7207 mg Microsomal Prtein/m Incubato: o3
e 53 wms72 mg Microsoma Protein/g Liver Wt =
£ w0000 o8] sz e Hepatic Hood Fion: EE
£ 684 o573 < Results
= 200000 o S s otReviened e -
000 837 o542 Intinsic Hep. Glearance (Ljr ) 199
.12 73933 Predicted Hep. Clearance (e 103
i, min , || Resuts Saved and ocked en 03]
N | LastPont Remainng (%) 63.1

B Smoots Y = (0.00208437X + (1.9168)

R2=020128 Chrom Data Used for Results

1

@ [ Swdhronize

c Aoaiyewis = e o[ ez
W C or R (2 paie 2720/150.1 [ E TN ) cosc27s || N o] 0.06esezses
Normaize Y| 155 106376.57 g 2] 007
53 898 sas e [230 sesss2 = 0] 0.0622213573]
230, 209 S 305 110607.77 g 2] 0.07%59512)
roces o |38t v : 12 o.0662229%6
& sooes o2 ¥ 456 11054718 5 [ o071l
= oo0se] 0a0 533 LomLen H T 001739
£ o mems || oo
e s o || % S oomses
= 2o 758 130457 K o o. 110593201
000l 77971.08 o e 0 s W@ o o.06%476279
i Time (Minuts)
Pt
o Reset || 6 ves || @ e

o € Reps

e, |





Figure 16
Loaded raw data file data

11.8 Choose a Sample

Users may choose a sample in the ‘Choose a Sample’ grid either by selecting with the mouse or using the arrow keys when the ‘Choose a Sample’ grid has focus.
If chromatograms and/or results have not been saved when the user scrolls through samples, the user will be prompted with warning messages to choose to save chromatograms or results. These warnings may be turned off be de-selecting the appropriate checkboxes located to the right of the ‘Choose a Sample’ grid (see Section 11.2).
If a sample is of incorrect format or if data file is of incorrect format, an error message will be displayed.

12 Hepatic Clearance Calculator – Chromatographic Processing
The Chromatography bottom left pane allows users to perform several functions:

· Allows users to quickly view analytical runs generated in one or several raw data files and quickly identify possible problems in instrument operation or sample preparation.
· Automatically optimizes chromatographic integration parameters and picks peaks.

· Allows the user to further optimize chromatographic integration, including manual integration.

· View Hepatic Clearance Results on the fly as chromatography is being optimized.

12.1 Display chromatography

There are several ways in which chromatographic data may be displayed:

12.1.1 Retention Time/Peak Area table grids
If the user clicks on an entry in the ‘Retention Time/Peak’ Area table grid located to the right of the chromatogram, the corresponding chromatographic peak will be zoomed in the chromatogram.
For example, Figure 17 shows that, when peak retention time 1.40 in the ‘Retention Time/Peak’ Area table grid is selected, the corresponding peak obtains focus in the chromatogram. In the figure shown, the ‘Synchronize Analyte w/IS’ checkbox was selected, so the corresponding Internal Standard peak was also obtained focus.
[image: image17.png]Hepat lculator
=
Hepatic Clearance Calculator Choose a Sample Expand
N o Tobutenide Toluamide 11 08,2006 17 4 —
Choase Data Access Confiuration. Choose Data Acgisiion Mode: Noheryion Noheryion BB TR |
@ Databased on a single data fe € Low Thioughput Acauisiion Dertonethoghen[Demtonsthogten[Wi06 2006 16 |~ Epeble Samdle Chnge Waring
" Data based on dala fes within adiectory | | & High Thoughput Acauisiion T extoterorn T extoterorn ITIERIEE

Enatle Sample Change Warring
T Resus

. [

[Er\- Proicts 000041200607 10_FiepGHTPIb pata 20060708 Kim Vmax wit | @im 3 | Cancel | Exi
BF Smooths  NT A& Chromatogr. Review Analyte: Time Point Set: [Mrosome ~
e [ores #peskss[ 13| [rthoxycouman o, s, [T =
G of +HIRM (2 pair) 191271073 Mex 720t cps. Analyte [575T 223109
Normaize Y| 1.54 155498 Results Pane pem—
2 ® ves 230] 8056.46) ‘Comments (<250 characters) Species: [pat Miaosame =]
rat o 305 7130 =
(S LI 7 o055 || g ofLver Weight/Kg Body W E
£ 35 se0L5t mg Microsomal Prtein/m Incubato: 3
. Sove || 535 205625 mg Microsoma Protein/g Liver Wt =
g | X I — Hepatk Bcod o33
2 =009 6.87| 38767.57
= ot Reviewe: " =] [ Results
" 763 3004405 ot Reviewed s =
o0 8.40| 71039.5 sz Intrinsic Hep. Clearance (Ljhr/kg): 26.5)
2:15|_ oa145.83] s Precicted Hep. Clearance (Lrfk): |
» eno.091]
| P B et | LastpontRemainng (0 360

= (00881137 + (18533

7

@ Synchvonize Chom Data Used for Results
¥ eal

c Analyte wiIs #Pesks: o] 0om7%iD]
50 of sHRM 2 pas): 2720/158.1 Mex 12850 putcrq [073] 9568639 ~ 0] 0.0233722010
oy 154w || & o[ o.068191027
250 el 230 weranst = 2] 0.075a750613]
Sves |05 sz g 12 o.155755%6
sost P H 2] o.1smeisis
£ oou s H T ozuzsre
z 535 109377.43 5 7] o0.2373050134
£ om oo mewa || o srsses
E o e || 2 o
- 7.63 119706.02 2 0| 0.7613623658]
004 8.40 7870378 o= ‘: (";' 7;“ = = o 0.5026187815|

T =7 28 35 w0 9.15 842513 s IS Pt Eeoce

Time, min o Reset || 6 ves || @ e —

Remove venualItegraton | [ [E[IE] meset | ot | P | Excude

udge =





Figure 17
Click on Peak 1.40 of Analyte Retention Time/Peak Area table grid (with ‘Synchronize Analyte w/IS selected
12.1.2 Synchronize Analyte w/IS

If the ‘Synchronize Analyte w/IS’ checkbox is selected, then a selection in either ‘Analyte –‘ or ‘Internal Standard Retention Time/Peak Area’ table grid will result in the corresponding peak obtaining focus in both chromatograms.

12.1.3 Normalize Y

· If Normalize Y is set to Yes, then the Y-axis is automatically normalized to the peak height.

· If Normalize Y is set to No, then the Y-axis remains at its most recent setting

· If Normalize Y is set to x10, the Y-axis is automatically magnified by a factor of 10 with respect to the peak height of the selected peak

12.1.4 Y-Axis Scale buttons
The gray arrow buttons to the right of the chromatograms increase or decrease the scale of the Y-Axis

12.1.5 X-Axis Shift buttons

The gray arrow buttons to the bottom right of the chromatograms shift the X-axis to the left or right.

12.1.6 X-Axis Expansion buttons

The blue arrow buttons to the bottom of the chromatograms expand or contract the scale of the X-axis

12.2 Integration Parameters

Users may optimize integration with the following parameters:
· Bunching Factor (BF)

· Smoothing (Smooths)

· Noise Threshhold (NT)

· Area Threshhold (AT)

12.3 Manual Integration

Manual integration may be performed by clicking and dragging (Panel 1) with the left mouse button the peak desired to be integrated. The corresponding item in the Retention Time/Peak Area grid will be annotated with an “M” to signify that the peak has been integrated manually (Panel 2).
	Panel 1
	Panel 2

	[image: image18.jpg]&

Smooths

o9

W00 of <MRM (2 psis): 4342111825

Intensity, cps

1000

g

o] .

2o 27 28 28
Time. min

kel <=

Chromatogr. Review

fora ]

e

€ Intstd

Normalze Y
CYes
C o
@ 10

save

B (

074
192
209

276 IEEZREM|

3.4
a1
a7
543
811
577
7.4
811

R
0669.13
21088.79

2172085
2547
242.92
233338
7781936
50086.30
5%669.63
002165

Al

[ [ aad




	[image: image19.jpg]B Smooths AT [ PantoPom | Chromatogr. Review
€ Dioptobsseine | [lorag -
W ot R (2 i) 434211935 Max 5504 o
1200
2 o
=
E o
o

@ Anayte
€ st

Normaize Y

CYes
Cho
@ w10

2 27 28 28
Time. min

Remave Manual Integraton l—l;

Save

I’
Reset

Chvom

2172085
2047
242.92
2333338
778195
50086.30
54659.63
002165







Figure 18
Manual Integration

The manual integration can be re-manually integrated if desired.

The manual integration may be removed by:

· ensuring that the correct peak is selected in the Retention Time/Peak Area grid

· clicking on the Remove Manual Integration button

12.4 Manual Peak Setting
In some instances (e.g. when peak heights are close to the signal-to-noise ratio or if integration parameters are not optimal), Hepatic Clearance Calculator will not identify the correct peak. The peak may be manually set by:
· ensuring the peak in question is selected in the appropriate Retention Time/Peak Area grid

· right-click on the desired peak

12.5 Manual Peak Retention Time Entering

In some instances, several peaks cannot automatically picked or cannot be set with a right-mouse click. The user may manually enter retention times for the run by choosing Menu – Manually Enter Peak Retention Times. See Section 15.6 for further discussion

12.6 Chromatography Review Dropdown Box

The user may choose from a list of choices to denote an assessment of the chromatographic results. The contents of the ‘Chromatogr. Review’ dropdown box are configured by an administrator in the Configuration Utility.

12.7 Save Chromatogram

When the user is satisfied that the chromatography is acceptable, the user locks the chromatogram by clicking on ‘Save Chrom’. This disables all functions relating to chromatographic integration and the ‘Chrom Saved’ checkbox becomes selected. In addition, chromatographic data are stored in the GMSU data store.

If the user attempts to further process a Saved Chromatogram, an error message will be displayed. In order to further process a Saved Chromatogram, the user must deselect the ‘Chrom Saved’ checkbox.

12.8 Batch Accept All Chromatograms

In order to expedite the processing of a large number of samples, the user may batch process and save all samples by choosing from the menu Menu – Accept All Chromatograms. The samples listed in the ‘Choose a Sample’ grid will be processed sequentially and application-optimized chromatographic data will be stored in the GMSU data store. See Section 15.1 for further details.
13 Hepatic Clearance Calculator – Results
The Hepatic Clearance Calculator Results bottom right pane allows users to perform several functions:

· Automatically generates Hepatic Clearance Results based on selected species-specific Experiment Constants.

· Allows the user to optimize Hepatic Clearance Results by selecting the choice of using or not using internal standard peak area ratios.

· Allows the user to optimize Hepatic Clearance Results by excluding data points.

· Allows the user to select Results Review assignments and comments.

The following describes the functionality of the Hepatic Clearance Calculator – Results section.

13.1 Overview

The right Results Pane contains functionality for users to optimize and process data to obtain Hepatic Clearance Results.

	
[image: image20.png]Results Pane Constants

‘Comments (<250 characters) spedesi[RatMiosome ]

‘gof Liver Vieight/Kg Body Wt |

mg Microsoml Protein/ml Incubation 5|
mg Microsoml Protein/g Liver Wt |
Hepatic lood Flow{ 3.3

Results Review

NotRevensd x| | Resuts
[l -

= Insinic Hep. Clearance (Lwka}] 26,9
Resus Predicted Hep. Clearance (Ljr/ko): 2.54)

e oasy]
LastPoint Remaining (%): 5.50)

= (00881137 + (18533

pasidl Chrom Data Used for Results

1

0.01575%6117]
0.023372200]
0.0651391027|
0.07547505.13]
0.14557954%4|
0. 15783514
0.29027157%|
0.2373090134]
05278360955
05254681921
0.7613623655|
0.3025157515]

Peak Area Ratio (Log(As))

1P 1 4 5 51 5

T 5 w1 = % ®
Time (Minutes)

Use s Plot: =

Reset || G ves || @ ave

o  Reps s





	· ‘Comments’ box allows users add comments about sample. Comments may be included in Reports.

· ‘Results Review’ box allows the user to choose a Review Status.

· ‘Save Results’ button allows users to save/lock the data such that it can’t be changed unless manually unlocked.

· ‘Results Saved and Locked’ checkbox allows users to unlock data for further processing.

· ‘Species’ dropdown box allows users to associate samples with a specific species

· The regression plot shows plotted data. Users may expand either axis by clicking/dragging the axis.
· The ‘Chrom. Data Used for Results’ shows the retention data, time point, and peak area or ratio data for each data point.

· ‘Reset’ button returns plot to full scale on both axes.

· ‘Use IS’ frame allows users to choose to use Internal Standard peak area ratios or not.

· ‘Plot’ frame allows users to display average or individual replicates in the regression plot.

· ‘Exclude’ and ‘Add’ buttons allow users to exclude and restore data points.


Figure 19
Hepatic Clearance Results Pane Overview

The following describes the functions available in the Hepatic Clearance Results pane.
13.2 Use IS Option Frame

Users may choose to use (Yes option) or not use (No option) internal standard area ratios by selecting the appropriate option button. The y-axis label will update accordingly.

	Use Internal Standard
	Don’t Use Internal Standard

	[image: image21.jpg]= (0 0388037 + (18779)

1

Peak Area Ratio (Log(A/1S))

a7ar

[lsels
@ g
o

W w28 A
Time (Minutes)
Add—>
Pl
@ dve Exclude.
<

€ Reps





	[image: image22.jpg]e

Peak Area (Log(A))

0ol

0385307 + 20128)
Rz =ng8ea2

[lsels
© ves
© o

L)
Time (Minutes)

Pl
& ave
€ Reps

Add >
Exclude.
P






Figure 20
Use IS Frame Options
13.3 Plot Option Frame

Users may choose to display average replicate points (Ave option) or individual replicates (Reps option). The individual replicates view is useful to identify individual outliers.
	Display Average Replicate Points
	Display Individual Replicate Points

	[image: image23.jpg]= (0 0388037 + (18779)

1

Peak Area Ratio (Log(A/1S))

a7ar

[lsels
@ g
o

W w28 A
Time (Minutes)
Add—>
Pl
@ dve Exclude.
<

€ Reps





	[image: image24.jpg]0038603 X + (19775)

E

Rz

Peak Area Ratio (Log{AS))

arar

[lsels
@ ves
€ o

L)
Time (Minutes)

o
e
@ kepd

% @

Add—>

Exclude.
<






Figure 21
Plot Frame Options

13.4 Constants Dropdown Box

Hepatic Clearance value calculations contain constants that are specific to species and assay. These constant values are configured by an administrator in Configuration Utility. The constants set for each sample is chosen by the user. If a user changes the species constant set, the Hepatic Clearance Values will update automatically.
	Results with Rat Microsome
	Results with Mouse Microsome

	[image: image25.jpg]Constants

 cF Liver WeightfKg Body W:

ma Microsamal Prateinfml Incubation
m Microsoml Proteinfg Liver Wi:
Hepatic Blood Flow

Results
ti2,

Intrinsic Hep. Clearance (Lfhvfka):
Precicted Hep, Clearance (Lfhvka):
Eh

Last Paint Remaining (%)

]

5]

|

53]

7]

214

250

0,567

655





	[image: image26.jpg]Constants
Species:

 oF Liver WeightfKg Body W:

ma Microsamal Prateinfml Incubation
m Microsoml Proteinfg Liver Wi:
Hepatic Blood Flow

Results
ti2,

Intrinsic Hep. Clearance (Lfhvfka):
Precicted Hep, Clearance (Lfhvka):
Eh

Last Paint Remaining (%)

7.5

5]

|

4]

7]

220

479

0.5

655







Figure 22
Species Constants Settings
13.5 Exclude Data Points

Users may exclude data points if desired by selecting a data point to exclude, then clicking on the ‘Exclude’ button. Results will be updated automatically.

For example, if the red 30 minute replicate in Figure 23 is to excluded, the user would select the red 30 minute time point in the ‘Chrom. Data Used for Results’ grid and click the Exclude button.
	Exclude Data Point
	Results Updated

	[image: image27.jpg]L——————————— 1 Results

Resuts Review [ —
NotRevewed %] | Intinsicep. Clearance (Uhrfla) g
Predicted Hep. Clearance (Ljhr/ka): 290

Save ehl o
MSJ Last Point Remaining (%) 544

Chvom. Data Used for Result

00421857 + (1.9798) 05377365382

R2=ng8s22

1

12757543509
14677223457
25063686187
3.1064679608
45017072815
5. 6737088509
7.126623109
50665093622
97279557428
114320234857

23

Peak Area Ratio (Log(A/1S)

ool

§ s w18 =z @
Time (Minutes)

Add >
[lsels Pl

Reset || @ ves || ave Exclude.
=

€ o @ Reps





	[image: image28.jpg]———— rResuits.

Resuts Review [
NotRevwed =] | IntinscHep. Clearance (Uhrfka) 27,3
Predicted Hep. Clearance (Ljhr/kg): 2.94)

Save ehl oo
M Last Point Remaining (%) 3.43

Chvom. Data Used for Resul:

0 s377sesa]

20| _1.2757549509)
20| _1.4677223497]
12] _2.5063866157)
12]_3.1064679606)
7|_a.5017072815,
7] _s.6737088509)
3| 71268231098
5| _s.0ees053622)
0

0

Y= (0 0435057 x + 2.0208)
R2=0.99440

1

57279537425
114320234857

PeakArea Ratio (Log(A/IS))

ool

¢ 8 w15 =z @

Time (Minutes)
add >

[lsels Pl

Reset || @ ves || ave Exchude.

o @ Reps







Figure 23
Excluding Points

Note: Users may exclude as many data points as desired given that two points are remaining to generate a regression

13.6 Restore Data Points

Similar to the Exclude procedure, users may restore excluded data points by selecting the data point to restore, then clicking on the ‘Add’ button. Results will be updated automatically.

13.7 Comments

Users may enter comments about the sample. The comments field may be selected for display in a user defined report (see Section 0).

13.8 Review

After optimization of Results data, users may classify the results by choosing a value for ‘Results Review’ dropdown box. The contents of the dropdown box include ‘Not Reviewed’, ‘Acceptable’, and ‘Unacceptable’.
Selecting the proper review value is important with respect to Reports. Users are allowed to filter Reports based on the ‘Review’ field contents.

13.9 Save Results

When the user is satisfied that Results processing is complete, the user locks the Results by clicking on ‘Save Results’. This disables all functions relating to chromatographic integration and results processing and the ‘Results Saved’ checkbox becomes selected. In addition, Results data are stored in the GMSU data store.

If the user attempts to further process Saved Results, an error message will be displayed. In order to further process Saved Results, the user must deselect the ‘Results Saved’ checkbox.

14 Hepatic Clearance Calculator – Reports

Users may generate Microsoft® Word or Excel reports based on user-defined choices. The reports module may be activated from the GMSU Console (see Figure 24) or from the Hepatic Clearance Calculator menu
 Menu – Generate Reports (see Section 15.1).

[image: image29.png]3 GMsU Console

Gubbs™Mass Spec Utilities

- Home

~ Hepatic
Clearance
Calculator

Hepatic Clearance Calculator
Hepatic Clearance Calculator Reports
Go Back





Figure 24
GMSU Console

The default field displayed in a Report include:

	· Analyte
	· Predicted Hepatic Clearance

	· Intrinsic Hepatic Clearance
	· Eh


Additional fields that may be displayed in a Report include:

	· Results Review (Acceptance Status)
	· Sample Number

	· Comments
	· Species Experiment


To generate a report:

· User chooses one or more Analyte-specific items and clicks the Add button to add them to the print queue.

· Users may enter an optional Print Job Description.

· User chooses a Results Type filter and any additional report content.

· User chooses a Report Format.

· User clicks Generate Report button

[image: image30.png]Reports

Hepatic Clearance Calculator Reports

Existing Data...

Sort
@ by Anate

€ byDataFle by Sample

Analyte

01 4455 0BLd HTI SDF)
Phenacetin

Phenacetin

Sample Raw Data File

:\Anslyst Data\Proieots\00004\20060710_HepCHT ProbADatah20060708_ K Vi wif

Add

Remove

Data to print.

Choose one or more entries in the above table.
‘Then click Add to add the entries to the orint aueue,  Enter aPrint Job descrption (Optional):

Sot Pint Data
@ by Anate
 byDataFie
by Sample

Choose Addiionsl Report Content - Filer/Choose Resuls Type
Re-S0rt | - Choose a Report Format—| | [~ Acceplance Status € Acceptable

@ Microsaft Excel I~ Comments € Unacceptable
Generate ||  Mistosoft word I~ Acquisiion Time € NotReviewsd

% Species Expeiiment @A





Figure 25
Hepatic Clearance Calculator Reports Module

Note: Samples that have a ‘Last Point Remaining %’ value of greater then the CutOff value will display in the Report special text values specific to a species and experiment (all configured by an administrator in the Configuration Utility). For example, if a sample is has a ‘Last Point Remaining %’ of greater than 90% and the species experiment is Rat Microsome, the Intrinsic Hepatic Clearance, Predicted Hepatic Clearance and Eh values would be ‘<0.85’, ‘<0.68’, and ‘<0.21’ (or some other configured text values).
The CutOff value is configured as ‘Last Point Remaining Value’ in the Hepatic Clearance Calculator window (see Section Error! Reference source not found.) of the Configuration Utility.

15 Hepatic Clearance Calculator – Additional Functions

Additional Hepatic Clearance Calculator functions are found under the menu Menu. The following sections describe these functions.
[image: image31.png]3 Hepatic Clearance Calculator
veru

‘Accept Al Chromatograms...
Change Data Fie Path.
Copy Peak Integratons to Clpboard
Generate Reports...

Peak Area TextFie...

Manually Enter Peak Retention Times.
print

Enter LT Retenton Time - Analyte
Enter LT Retenton Tme - Int Std
Display Multple Results.





Figure 26
Menu Items

15.1 Accept All Chromatograms

The ‘Accept All Chromatograms’ function will batch process and save all Chromatographic and Hepatic Clearance Results for all samples displayed in the ‘Choose a Sample’ grid. This is a quicker way to review and accept data and is useful if the user knows that chromatography was most likely sufficient.

15.2 Change Data File Path

15.3 Copy Peak Integrations to Clipboard

15.4 Generate Reports

‘Generate Reports’ will display the Reports window shown in Figure 25.

15.5 Peak Area Text File

The ‘Peak Area Text File’ function will export Analyte and Internal Standard time point and peak area data to a comma-delimited text file. The text file header includes ‘Date of Report’, ‘Raw Data File Path’, ‘Analyte Name’, and ‘Acquisition Time’.
15.6 Manually Enter Peak Retention Times

There are times when the automatic peak identification algorithm of GMSU will not identify peaks properly. This typically occurs when a peak or entire chromatographic signal is close to a signal/noise of approximately two. Users may use the ‘Manually Enter Peak Retention Times’ function to manually enter the expected peak retention times or generate and apply a new chromatographic method.
[image: image32.png]A peak Method

Enter Expected Peak Retention Times:

Peak

Analyte RT

Int St RT

Peak 1

Peak2

Peak3

Peak 4

Peaks

Peakb

Peak 7

Peak 8

Peakd

Peak 10

Peak 11

Peak 12

Apply Ret Data to Chromatogram:

€ Anate

€ Intemal Standard

@ Bath

Chromatogram

Save this dataas a
Chiomatogaphic
Methad.

Apply to

Exi

Obtain Retention Data from:
© g0
 Chomotogratic ethod

Use Ret Data from:

© Anate € InlemalStandard @ Bath

Select 3 Sampl toretieve retertion data

D Da
08, 2006 17.38.02

Phenacetn





Figure 27
Manually Enter Peak Retention Times

Users may choose to enter retention time for Analyte, Internal Standard, or both, in the left portion of the window. If desired, the user may save this retention data as a new chromatographic method by clicking the ‘Save this data…’ button.
If desired, the user may use retention data from an existing saved sample by selecting the ‘Existing Data’ check box located in the ‘Obtain Retention Data from:’ frame. If the user checks this box, then clicks on a sample in the ‘Select a Sample to retrieve retention data’ list, retention data from that sample will be automatically entered into the appropriate boxes (see Figure 28).

[image: image33.png]A peak Method

Obtain Retention Data from:
Enter Expected Peak Retention Times: @ Evisting Data

Peak | Analyte RT | Int St RT € Chiomologiaphic Method
Pesk 1 0.0 00
Pesk2 155 155
Pesk 3 230 230
Pesk 4 30 35
Pesks 381 381
Peskb 459 459
ey 2 539 Use Ret Data from:

Peak 8 6.06] 6.05]
Feokd God] o] © Anate € InlemalStandard @ Bath

Peak 10 760] 759
Foak 11 5771 53| Select 3 Sampl toretieve retertion data:

Peak 12 512] 312) Date

Apply Ret Data to Chromatogram:  Save is daa as
Chomatographic

 Anapte et

€ Ineml Standard

@ Both Apply to
Chromatogram

Exi





Figure 28
Use peaks from an existing sample

Alternatively, the user may wish to apply retention data from an existing chromatographic method by selecting the ‘Chromatographic Method’ check box located in the ‘Obtain Retention Data from:’ frame. If the user checks this box, then clicks on a method in the ‘Select a Method to use’ list, retention data from that sample will be automatically entered into the appropriate boxes (see Figure 29).
[image: image34.png]A peak Method

Enter Expected Peak Retention Times:

Peak

Analyte RT

Int Std AT

Peak 1

4.18]

478]

Peak2

523

523

Peak3

6.06]

6.05]

Peak 4

71

71

Peaks

75|

75|

Peakb

5.00]

500

Peak 7

95|

95|

Peak 8

108

108

Peakd

11.75

11.75

Peak 10

1283

1283

Peak 11

1359

1359

Peak 12

1458

1458

Apply Ret Data to Chromatogram: ™ Gave his data s &

€ Anate

€ Intemal Standard

@ Bath

Chiomatogaphic
Methad.

Apply to
Chromatogram

Exi

Obtain Retention Data from:
€ Esising Data

@ Chiomotographic Method

Create Method Parameters:
Enter Expected Nunberof [—5 ] GetRetertion Data fom
Peaks: s o Excel il

Use Ret Data from:

© Anate € InlemalStandard @ Bath

Select a Methad to use:

Method Name
Andreas_10

Andreas 14
Andreas_ 16
Andreas_16_01
mic20060630
prac?





Figure 29
Use peaks from an existing chromatographic method

After retention data has been entered:

· the user clicks on the ‘Apply to Chromatogram’ button

· the retention data is entered into the appropriate chromatographic data grids

· the GMSU integration algorithm is called to calculate peak areas

15.7 Print

The print function will print the entire window as shown.

This function is not available at this time.

15.8 Enter LT Retention Time – Analyte

This function is used to set the expected retention time of the analyte for LT data.

15.9 Enter LT Retention Time – Int Std

This function is used to set the expected retention time of the internal standard for HT data.

15.10 Display Multiple Results

This function will use MS Excel to plot and print regression results of several analytes on one regression plot.

This function is not available at this time

16 Permeability Calculator

The Permeability Calculator section is under construction. At this time, figures will be shown.

[image: image35.png].0

[ Swdhronize

= Peaks:

Analyte w/S
W o @ pa 272002 15 Ve 8405 oo[fTT
Int Std
8.0e4: xic
Chomoize
g oo e
Z o O o
2 @ ¥10
R

et

7543180
5166%.55
713%62.36
51415111

7707%8.33
7 1367595.80
5 s461L61
5 1141917.72

10 s%27.75

11 1070944.05

12 550550.75

P I = o

N

Use s
@ ves
o

add—>

Permeability Calculator ‘Deﬁu\tPeakName set Cﬁseasamv\e
Coco? LT 55 5
Choose Dt Acausion Mode: Epable Sanple
@ Low Throughput Acquisition Change Warning
 High Thoughut Acauision
Diply
choose a Raw Data Fie: _ Bromse.._ | Chiomatogiaphy. €im > Exi
[c:Vnalyst Data Projects AA\Caco_20060306 e 20060221_Caco2_CCO0107_| Fonife Cancel
Chromatography! I Reslts ) Calibration
BF  Smooths  NT AT [ Chromatogr. Review Analyte: [ 1| Comments (<250 characters)
(1] - ioreg ress (5| s |
W i oA @) <56 25801 ex 25200 ST T e | Peskiane se
Anayte | 2 us09 || -
. Pl e
. ormalze v| 178557
& w0 © Yes s
2 O o 2657
£ Sl |7 asenn
§ 0 8 1364.68
= 02 053 s Ba3.70
1o 572.3
11 181184.11  Weighting Y =(0.00013032)°X + (-0.0017038)
12| 134094.76 [1(none] R2=088830

Peak Area Rao

o 200 40 o0 80 1000

(Calibration Data

BT T Conc|  Response | Calc Conc| %Bias | ~

2| o.001712013] %2 w0l |

Excude 0] 0.0021934555) 2.9 1390] |
< 55| 0.00678706%3] 65.2] 18.5]
266 _0.0319558131] 255 50
998 _0.1510773724 11| 1.1

2 _0.0005523945 73] Z65.0] ¥





Figure 30
Permeability Calculator – Chromatography Tab

[image: image36.png]Menu

Permeability Calculator piaitPektinese: e sarve
Choose Data Acquisition Mode: o

@ Low Thioughput Acapisiion
 High Thoughput Acquisiion

Display
Choose a Raw Data Fie: _ Browse. Chiomatography. @i )

Enable Sample
Changs Warning

‘C :\Analyst Data\Projects\AA\Caco_20060306\New\20060221_Caco2_CC00107_MLO0649843.wiff Cancel
Chromatography Y Results I Calibration
Receiver Side: DA -> RB
[ CDaoRB T DE-RA @ Boh Descr. | Ave | SEM |Exi] Rep1 |Ex2| Rep 2 [Ex3] Rep3 | | Excuce ‘
Tomin [Te1912z]7182007  [198108g] [Te701] [1.729008) | _Fonis
20080221, Cacoz ceootorl ] 30 min 4.101902] 0.24840€] 459852 3841717 3.86546¢]
r (<8 ) >| 45 min 300687 0.937160__|10.18931] | 7080221 | 751252
o ~| [60min —[1521964 1.350447 1703845 1538506 1200861
Comments (<250 characters) Data_| Conc. RTime |Anal | (Slope =~ 17.76814]1.770407] 20.88117| 17.6727C] 14.75057]
Blank | 3024537 1 1| [R2 0.945317] 0.960982] 0.910307| 0.96464¢]
Blank| 30,0360 2 1 | [Papp 0747817 7.45121C] | 0.878837] | 0.74380C]
A5t 1[ 2621087 51 | [Slopeze [z2238]
A5t 2] 2890567 i1 | [Papp2or [0ssmez] g
] [ETEIA 57 | Receerside: 06 > va 0% e = proeiin
‘Select an Assay Time Point Set ASW5[1172.351 T3 Desci. | Ave | SEM [Ex1| Rep1 [Ex2[ Rep 2 [Ex3] Rep3 Excude
Defalt H (e s | [TEmin_[7s197970 716257 [mesorsd [eiab [67oze | _pons
o S a | [20min [2i7svecloamen [17.2m2 |osested] [2zdmne
Select a Papp Constant Set: | o e | [45min[s7rasedeteso Tatizen |sase| err7red
oetaat H [mozm e | [B0min[7assore|70mse] es00ase] |792860e] [sn3erre
oo e Bt o e e e s B
Basel Dosing (ml) 01| [_DA60) tz72e3 13 184 _ g | 2
Concentration (ubole/L] 20| |_DA-60, 128.004C] 14 13 Papp’ 4183075/ 0 460821 3389992 4173000
Growth Area (em 2) .33 |__DB-0] 191.02% 15[ 21 | [Slope2pt 1135860
Sampling from & (mL] 012 DB-0] 193.8177| 16[ 17 Papp-2pt_|4.78055°] [ |
Samping from B (L] 02l ¢ 7] T @ Summary
Dosing Solution Volume (L] M e 7 T Papp (4-points) Papp (2-points)
Dummgnvmume (mL) e 012| (TR Da be (x10° cm/s) sEM(x106)  (x106 cm/s) MB (%)
Calc. Conc. Dx-0 | 1288.785 |1580.217 (x: (x10C cm/s)
Calc. Conc. Dx-60 [1074136 [1835.502 DA-to-RB| 0.74] 0.4 0.93]  Apical 333 833
Conc. (uM) 1538461571 5384615.) DB-to-RA 4183 0.5| 4.781| Basal 119.3] 116.2
nmole 4 Ratio 54 5]
Calc. nmole D0 3350841 (0B/DA
Calo. nmole D60 [279290%





Figure 31
Permeability Calculator – Results Tab

[image: image37.png]Permeability

Meru

Permeability Calculator ‘Déﬁu\gPLeTakstame Set: Cﬁseasamv\e
Choose Data Acauision Mode: - - Enable Sample
@ Low Thioughput Acguisiion Change Warring
© High Thoughput Acaisiion

Display
[c:¥inalst Data Projects\AA\Caco 20060306 ew R0060221_Coco? CCOO107] e ]
Chromatography ! Results Y Calibration
Chromatography Calibration Values Caco Calibration Values Caco Peak Name Sets

Apply to Chromatography

Time Points Peak Name Set Peak Names

Enter umber o Cabraton Fonts:
N 15 1 1|RB-20 Set204060
Concentrations Set 204060 30 168 2[RB-20 | Set204060
Current 306080120| 45 27B_5Calibr 3|RB-20 Set204060
o 3 Fract ] 276 bCalibr | |_4]RE-40 | Seto04060
4BAB_5Calibr 5|RB-40 Set204060
Std 1 2| 4BAB_Prac0l 6|RB-40 Set204060
suz 10 52A8_Callbr | |_7|RE60 | Set204060
Std3 55| 54AB_2 8|RB-60 Set204060
54AB_5Calibr 9| RB-60 Set204060
Std 4 266 8888888 10|Blank. Set204060
S5 939 339 a8az 11|B-5td 1 Set204060

Coco? HT 55| [ 12[E-Std? |Setz04060
Coco2 LT 55| [ 13[E-5td 3 [Set204060

Test 14[Blank__|Set204060
Testl 15[DAE0 | Set204060
22z 16]DAE0 | Set204060
222722 17]DA0 | Set20d060

222222227 18]Blank | Set204060





Figure 32
Permeability Calculator – Calibration Tab

17 References

1 R. O. Cole.  Quantitation of Biofluids with High Speed HPLC/MS/MS:  Early Discovery Applications.  Invited Presentation, at “Early ADME and Toxicology in Drug Discovery: Techniques for accelerating and Optimizing Drug Candidate Selection”, Berkley, CA, October 1998.

2 Adam H. Brockman, Donna L. Hiller, and Roderic O. Cole. High Speed HPLC/MS/MS Analysis of Biological Fluids in Support of ADME Screens: A Practical Review. Current Opinion in Drug Discovery, 4(3), (2000), 432.

3 Janiszewski JS, Rogers KJ, Whalen KM, Cole MJ, Liston TE, Duchoslav E, et al: A high capacity LC/MS system for the bioanalysis of samples generated from plate-based metabolic screening. Anal Chem 2001; 73:1495-1501.

4 Z. Yan, J. Wu, L.E. Elvebak, A. Brockman: Validation of a Totally Commercially Available High Throughput -ADME System and Results for 60 Literature Compounds. Rapid Communications in Mass Spectrometry, Rapid Commun. Mass Spectrom. 2005; 19: 1191–1199.
Version 6.0 (July 20, 2006)
C:\GubbsIncApps\GMSU\UserManuals\UserManual\GMSU_UserManual_06.doc

[image: image38.jpg]_1214918632

